MOST IMPORTANT COMMANDS YOU MUST MEMORIZE

## PC Commands

ipconfig

ping 192.168.1.1

tracert 192.168.2.1

## Router Commands

enable

configure terminal

interface gig0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface gig0/1

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

ip route 192.168.3.0 255.255.255.0 10.0.0.2

show ip route

show running-config

show ip interface brief

## Switch Commands

enable

show mac-address-table

show running-config

# IMPORTANT TOPOLOGIES

## Star Topology

- One switch in center

- All PCs connected to switch

- Most common in Packet Tracer

## Bus Topology

- Single backbone cable

- Rare practically in Packet Tracer

- Mostly theoretical

## Ring Topology

- Devices connected in circular form

## Mesh Topology

- Multiple connections between devices

- Very reliable

# POSSIBLE MID EXAM QUESTIONS

# QUESTION 1 — BASIC STAR TOPOLOGY + PING

## Task

Create a star topology using:

- 1 switch

- 4 PCs

Assign:

PC1 → 192.168.1.2

PC2 → 192.168.1.3

PC3 → 192.168.1.4

PC4 → 192.168.1.5

Subnet → 255.255.255.0

## Requirements

- Connect all PCs with copper straight-through cable

- Verify connectivity using ping

## What Teacher Checks

- Correct topology

- Correct IP assignment

- Successful ping

# QUESTION 2 — CLIENT SERVER NETWORK

## Task

Create:

- 1 switch

- 1 server

- 3 PCs

Assign:

Server → 192.168.10.1

PC1 → 192.168.10.2

PC2 → 192.168.10.3

PC3 → 192.168.10.4

## Requirements

- Connect all devices

- Ping server from all PCs

# QUESTION 3 — DHCP SERVER CONFIGURATION

## MOST IMPORTANT QUESTION

## Task

Create:

- 1 router

- 1 switch

- 4 PCs

Router acts as DHCP server.

## Router Configuration

enable

configure terminal

interface gig0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

ip dhcp pool LAB

network 192.168.1.0 255.255.255.0

default-router 192.168.1.1

dns-server 8.8.8.8

exit

## PC Configuration

Desktop → IP Configuration → DHCP

## Expected Result

All PCs automatically get IPs.

# QUESTION 4 — MONITOR SWITCH TRAFFIC

## Task

- Create star topology

- Send ping packets

- Use simulation mode

- Observe traffic

## Teacher May Ask

- MAC address table

- ARP requests

- ICMP packets

## Important Command

show mac-address-table

# QUESTION 5 — FTP SERVER CONFIGURATION

## VERY IMPORTANT

## Task

Create:

- 1 server

- 2 PCs

- 1 switch

## IPs

Server → 192.168.1.10

PC1 → 192.168.1.2

PC2 → 192.168.1.3

## Steps

### On Server

Services → FTP → ON

Create:

username: admin

password: 123

## On PC

Desktop → Command Prompt

ftp 192.168.1.10

Enter username/password.

# QUESTION 6 — TWO ROUTERS STATIC ROUTING

## MOST EXPECTED QUESTION

## Topology

PC --- Router1 --- Router2 --- PC

## IP Scheme

### LEFT NETWORK

PC1 → 192.168.1.2

Gateway → 192.168.1.1

### RIGHT NETWORK

PC2 → 192.168.2.2

Gateway → 192.168.2.1

### ROUTER CONNECTION

R1 → 10.0.0.1

R2 → 10.0.0.2

# Router 1 Config

enable

configure terminal

interface gig0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface gig0/1

ip address 10.0.0.1 255.255.255.252

no shutdown

exit

ip route 192.168.2.0 255.255.255.0 10.0.0.2

# Router 2 Config

enable

configure terminal

interface gig0/0

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

interface gig0/1

ip address 10.0.0.2 255.255.255.252

no shutdown

exit

ip route 192.168.1.0 255.255.255.0 10.0.0.1

# TEST

From PC1:

ping 192.168.2.2

If ping works → configuration correct.

# QUESTION 7 — SUBNETTING QUESTION

## Example

Given:

IP = 192.168.1.0/27

Find:

- Network ID

- Broadcast ID

- Number of hosts

## Quick Formula

2^n - 2

Where:

- n = host bits

For /27:

32 - 27 = 5

2^5 - 2 = 30 hosts

# QUESTION 8 — DIAGNOSING NETWORK ERRORS

## Teacher May Intentionally Break:

- Wrong subnet mask

- Wrong gateway

- Interface shutdown

- Wrong cable

## You Must Check

show ip interface brief

If interface is:

administratively down

Use:

no shutdown

# QUESTION 9 — DIFFERENT TOPOLOGY USING 2 ROUTERS

## Possible Variation

Create:

- Mesh topology

- Ring topology

- Star topology

Using:

- 2 routers

- 2 switches

- 4 PCs

Then:

- Configure static routing

- Test connectivity

# QUESTION 10 — COMPLETE COMBINED QUESTION (VERY POSSIBLE)

## FULL LAB QUESTION

Create a network using:

- 2 routers

- 2 switches

- 1 server

- 4 PCs

Requirements:

- Use star topology

- Configure static routing

- Configure DHCP

- Configure FTP server

- Test connectivity

- Verify using ping

This is the most complete form of your syllabus.

# MOST IMPORTANT THINGS TO REMEMBER

## 1. ALWAYS DO THIS

no shutdown

## 2. ALWAYS SET DEFAULT GATEWAY

PC gateway must be router IP.

## 3. STRAIGHT CABLE

Use:

PC ↔ Switch

Switch ↔ Router

## 4. CROSSOVER CABLE

Use:

Router ↔ Router

(Some Packet Tracer versions auto-select.)

# MOST IMPORTANT COMMANDS FOR MID

## Router

enable

configure terminal

interface

ip address

no shutdown

ip route

show ip route

show ip interface brief

## PC

ping

ipconfig

ftp